## Notes to the reviewer

- This document only covers testing the TAPI provider (NBI). The focus of the document is streaming. There should be an equivalent TAPI client (SBI) test plan.

- TAPI 2.6 has been used to construct the tests. There should be no significant changes to adjust this to test TAPI 2.5. To test 2.1.3 a few changes will be required to details, but not to the main body of the tests.

- It is proposed that testing of streaming operation, independent of content, uses that alarm stream as this is the most likely to be supported.

- Testing of the alarm stream content will be challenging as there is no mandatory RESTCONF GET operation to retrieve active alarm (UC17f was introduced after TAPI 2.1.3), hence the stream detail cannot be confirmed against the detail provided by the GET. A more manual validation will be required (somewhat like that used for the RESTCONF GET tests).

- Note that connect to “latest” stream capability is not well described in TR-548. TR-548 needs to be updated (see #645). When a request to start from latest is received the server will stream the next log record that arrives (i.e., the latest update) and then stream all records from that point. The stream will continue to operate as usual with compaction etc.

- TAPI Streaming Authentication Token structure is covered in TR-548 by example. It would be preferrable that a standard approach be used. Standardisation in this area is challenging. We should explore for opportunities (#652).

- We do not need to deal with dynamic stream addition/removal in the initial testing phases.

- The Pong behaviour description needs to be improved (#646)

- Test set-up and response sample detail needs to be provided so the validator can be written appropriately. The sample does not need to be in the test case detail.

## TAPI Streaming test plan (Second Draft)

The priorities set out in this test plan are relative to the overall priority of TAPI streaming. If streaming is supported then the capabilities tested in P1 tests are mandatory, P2 are highly desirable and P3 are of low priority.

This test plan tests a TAPI provider (NBI). There will be a separate test plan for a TAPI client (SBI).

The following tests align with sequences set out in Figure 32 and Figure 33 of TR-548 3.2  (reproduced here for clarity).

The test plan has been designed to test a set of stream configurations. The provider may only support a subset of streams. This should be declared by the development company supplying the provider prior to the testing.

The initial stream testing will take a specific stream and validate the operation of a stream. It will not validate the content.

The following sets of stream support will be considered for stated technology coverage and depth of model as per TR-547 tests:

- Alarm

- Topology, node, NEP, link

- CEP, connection (covered as the baseline stream)

  - Including physical route if supported

- SIP, connectivity services

- Device, Equipment, Equipment Holders

- Access Ports

  - Span and abstract strand if supported

- Path and path set

- PMs with sufficient oam (with stated depth of coverage)

- OAM (with stated depth of coverage)

TR-548 does not constrain streams to have any particular combination of contents so a solution may separate out each object type onto its own dedicated stream or it may have one stream with everything in it etc.

Most of the streaming tests will be performed on the same network as the TR-547 tests were performed on (assume this unless stated otherwise in the test).

Note that a test that yields entity detail consistent with results from the corresponding GET based test in TR-547 will pass even if the corresponding TR-547 test failed on some content issue. The tests here are not on specific content but on content retrievability and consistency with the RESTCONF interface.

Note that whilst the use cases are geared for a full stream solution, there is also an option to start the stream from latest and to use it as a notification channel (activated during RESTCONF alignment process using GET).

The company delivering the TAPI provider should indicate whether SSE or WS or both are supported. If both are supported, the test should be run fully for one of the supported techniques. A lighter set of tests can be performed for the other technique, to reduce test volumes, as the backend can be assumed to be the same.

| Test number | Category | Priority | Description | Further comments |
| --- | --- | --- | --- | --- |
| 2.9.1 | TAPI Streaming | P1 | Use Case ST-0.1: Get Auth Token | The response in 7.11.3 says “similar to”. This test plan assumes that definition in 7.11.3 is preferred. The vendor will explain their approach and this will be tested. |
| 2.9.2 | TAPI Streaming | P1 | Use Case ST-0.2: Discover supported and available streams | The company delivering the TAPI provider should indicate which streams are supported. The test should validate that all streams identified by the company are reported appropriately. The test includes RESTCONF GETs on the yang path /tapi-common:context/tapi-streaming:stream-context/tapi-streaming:supported-stream-type/tapi-streaming:connection-protocol-details/tapi-streaming:allowed-connection-protocols (note that supported-stream-type is a list with uuid as key). See Yang Tree-ST-0.2 Implementations should retrieve the stream-context object with fields=supported-stream-type(uuid) Note the difference between supported-stream-type and available-stream. In such case stream-type properties can be obtained with a GET in the supported-stream-type=uuid referred to by supported-stream-type-uuid The list of available streams should be obtained from the Yang path /tapi-common:context/tapi-streaming:stream-context/tapi-streaming:available-stream where available-stream is a list indexed by a uuid. Implementations should retrieve the stream-context object with fields=available-stream(uuid) For all but PM streaming , the support should be LOG_STORAGE_STRATEGY_COMPACTED, RECORD_TRIGGER_ON_CHANGE with LOG_RECORD_STRATEGY_WHOLE_ENTITY, LOG_RECORD_STRATEGY_CHANGE_ONLY or both. The supported-stream-type detail should indicate whether CONNECTION_PROTOCOL_WEBSOCKETS or CONNECTION_PROTOCOL_SSE or both. For TAPI 2.6 supported-stream-type should include CONNECTION_PROTOCOL_GNMI The streams should be seen to be in an appropriate state. For PM streaming the support should be LOG_STORAGE_STRATEGY_TRUNCATED, RECORD_TRIGGER_PERIODIC, LOG_RECORD_STRATEGY_WHOLE_ENTITY. The supported-stream-type detail should indicate that the stream is CONNECTION_PROTOCOL_GNMI with ENCODING_FORMAT_PROTOBUF. |
| 2.9.3 | TAPI Streaming | P1 | Use Case ST-0.3: Connect to Stream and align - new client (specific stream) | This test assumes that the relevant stream type is included in the supported-stream-type structure, that the actual stream instance is already in the available-stream structure and that it is running. This Use Cases should be validated for all streams separately (see later test detail) In this test cases a specific stream is chosen to validate the detailed operation is correct. It is proposed that the alarm stream (as specified in TR-548 section 4) is used. The precise sequence of the use case should be tested. The stream connection should be made using the address for the stream received in ST-0.2. The test client behaviour is assumed to be as per the use case. The network and client will be scaled so as to ensure that steps (6) and (7) in the use case can be successful. If the solution supports LOG_RECORD_STRATEGY_CHANGE_ONLY for some streams, then one will need to be chosen to run this test. For CHANGE_ONLY testing the stream content will need to be appropriately set up. This will require careful network preparation with at least one property recently changed from a prior known state. Another known property will also have been changed from the created state some while before the tests are performed such that it is in the compaction zone. A valid operational usage is to start the stream from latest. This should also be tested in this test cycle, although it is currently P3 as there is not a complete description in TR-548 of this specific usage. This test should be performed with a quiescent network exhibiting zero change such that the stream is silent at the end of the test. This test just validates the overall operation of the stream. The actual content of the stream will not be validated by this test. The content is validated in a later test. |
| 2.9.4 | TAPI Streaming | P1 | Use Case ST-0.4: Client maintains idle connection | The test client sends a pong frames to maintain the connection. The client stops sending pong frames, the connection should be closed by the provider requiring full reconnection. |
| 2.9.5 | TAPI Streaming | P1 | Use Case ST-0.3: Connect to Stream and align - new client (all streams) | This test assumes that the stream is already in the supported-stream-type structure, that it is already in the available-stream-structure and that it is running. Validate content for all streams for all supported entities. Validation is performed by comparing the stream output with the information from the corresponding RESTCONF GET. It is assumed that the RESTCONF validation has successfully proven the correctness of the data structures with respect to the network etc. It is not intended that this test validates the correctness of the content wrt the network, the test is successful if the content is the same as that derived via RESTCONF (accounting for the distinction between the RESTCONF tree retrieval and the stream entity propagation. The test is applied for all supported technologies (as the streams will provide data on all). A stream that produces data that is essentially the same as that from the corresponding RESTCONF GETs is considered as passing the streaming test (i.e., even if the RESTCONF data has errors in it. - Alarms (P1) note that there may be no RESTCONF comparison hence the alarms must be validated against those known to be in the network) - Topology (P3) As there is only one topology - Node (P2) - NEP (P2) - Link (P2) - CEP (P2) - Connection (P2) - SwitchControl (P3) - SIP (P2) - connectivity services (P2) - Path (P3) - path set (P3) - Device (P2) - Equipment (P2) - Equipment Holders (P2) - Access Ports (P2) - PMs (P3) with sufficient oam (with stated depth of coverage) - Oam (P3) (with stated depth of coverage) |
| 2.9.6 | TAPI Streaming | P1 | Use Case ST-0.7: Short loss of communications | To be described. |
| 2.9.7 | TAPI Streaming | P1 | Use Case ST-0.9: Client requires realignment | To be described. |
| 2.9.8 | TAPI Streaming | P2 | Use Case ST-0.5: Provider delivers event storm (or slow client) – bad day | To be described. |
| 2.9.8 | TAPI Streaming | P2 | Use Case ST-0.6: Provider delivers extreme event storm (or very slow client) – very bad day | To be described. |
| 2.9.10 | TAPI Streaming | P2 | Use Case ST-0.8: Long loss of communications requiring realignment | To be described. |
| 2.9.8 | TAPI Streaming | P3 | Use Case ST-1.1 Provider initializes and operates a stream | To be described. |
| 2.9.12 | TAPI Streaming | P4 | Use Case ST-1.2: Provider recovers a stream after internal loss | To be described. |

Yang Tree-ST-0.2

module: tapi-common
  +--rw context
     +--rw tapi-streaming:stream-context {tapi-streaming}?
     |  +--ro tapi-streaming:supported-stream-type* [uuid]
     |     +--ro tapi-streaming:stream-type-name?              string
     |     +--ro tapi-streaming:record-retention?              string
     |     +--ro tapi-streaming:segment-size?                  string
     |     +--ro tapi-streaming:stream-type-content*           tapi-common:object-type
     |     +--ro tapi-streaming:log-storage-strategy?          log-storage-strategy
     |     +--ro tapi-streaming:log-record-strategy?           log-record-strategy
     |     +--ro tapi-streaming:record-trigger?                record-trigger
     |     +--ro tapi-streaming:uuid                           uuid
     |     +--ro tapi-streaming:name* [value-name]
     |     |  +--ro tapi-streaming:value-name    string
     |     |  +--ro tapi-streaming:value?        string
     |     +--rw tapi-streaming:compacted-log-details {tapi-streaming}?
     |     |  +--ro tapi-streaming:tombstone-retention?              string
     |     |  +--ro tapi-streaming:compaction-delay?                 string
     |     |  +--ro tapi-streaming:max-allowed-segment-roll-delay?   string
     |     |  +--ro tapi-streaming:max-compaction-lag?               string
     |     +--ro tapi-streaming:connection-protocol-details {tapi-streaming}?
     |     |  +--ro tapi-streaming:allowed-connection-protocols*   connection-protocol
     |     |  +--ro tapi-streaming:encoding-format?                encoding-format
     |     +--ro tapi-streaming:information-record-strategy {tapi-streaming}?
     |        +--ro tapi-streaming:record-suppression?                      record-suppression
     |        +--ro tapi-streaming:value-expectation?                       value-expectation
     |        +--ro tapi-streaming:allowed-dither-from-value-expectation?   value-expectation-dither

Yang Tree-ST-0.2b

module: tapi-common
  +--rw context
     +--rw tapi-streaming:stream-context {tapi-streaming}?
     |  +--ro tapi-streaming:available-stream* [uuid]
     |  |  +--ro tapi-streaming:connection-address*      string
     |  |  +--ro tapi-streaming:stream-state?            stream-state
     |  |  +--ro tapi-streaming:supported-stream-type
     |  |  |  +--ro tapi-streaming:supported-stream-type-uuid?   -> /tapi-common:context/tapi-streaming:stream-context/supported-stream-type/uuid
     |  |  +--ro tapi-streaming:stream-id?               string
     |  |  +--ro tapi-streaming:connection-protocol?     connection-protocol
     |  |  +--ro tapi-streaming:uuid                     uuid
     |  |  +--ro tapi-streaming:name* [value-name]
     |  |     +--ro tapi-streaming:value-name    string
     |  |     +--ro tapi-streaming:value?        string

## Test Plan: TAPI 2.6 Streaming Use Cases (TR-548 v3.2)

1. Test Plan Overview

- Objective: Validate the TAPI Streaming Service functionality, progressing from basic connection handling to advanced compacted log behaviour.

- Scope: Covers all streaming (for WS, SSE and gNMI)

- Version Under Test: TAPI 2.6.0 / TR-548 v3.3

2. Test Environment

- TAPI-enabled SDN controller

- TAPI client/emulator (ST Client)

- Network simulator and/or physical network

- For specific use case, auxiliar systems (e.g. Auth endpoint). Mock-up or simple systems may be used.

- Specific test client streaming validation

3. Traceability to TR-548

Each test case is mapped to specific use cases defined in TR-548 v3.3. For detailed descriptions and sequence diagrams, refer to the corresponding sections.

Test Details

✅ Test Case 2.9.1: Use Case ST-0.1: Get Auth Token

Description / Purpose

This test case validates that a client can obtain an authentication token from the provider's security layer in order to access TAPI Streaming services (e.g., gNMI or TAPI Notification API). This step ensures that the system enforces secure access and that the provider supports token issuance using standard or configured authentication protocols (e.g., OAuth 2.0, JWT, Basic Auth leading to token issuance).

Prerequisites

- The authentication server (e.g., OAuth2 token endpoint) must be active and reachable.

- The client must possess valid credentials (client ID/secret, username/password, or certificate, depending on auth mode).

- The provider should be configured to issue tokens for the specific scope. Tokens may have a finite validity (expiration date) and short lifetime, so new tokens should be retrieved as needed. The usage of invalid or expired tokens should be flagged.

Procedure

- Construct Authentication Request
Using REST or other supported mechanisms (e.g., HTTP POST to /token), the client submits credentials to the provider’s authentication endpoint.

- Observe the Response:
The response should include a valid access token, its expiry, and optionally a refresh token or token type.

Expected Results / Pass-Fail Criteria

| Step | Expected Outcome | Result |
| --- | --- | --- |
| 1 | HTTP response code is 200 OK | ✔️ Pass if 200 received |
| 2 | Response body contains a well-formed access token (access_token), with expires_in and optionally scope | ✔️ Pass if token is usable |
| 3 | Token is accepted by the TAPI Streaming endpoint in follow-up tests (used in Authorization header) | ✔️ Pass if streaming request is authorized |
| — | Any error response (e.g., 401, 403, malformed token) | ❌ Fail |

A successful test validates both the token issuance and its functional usability in downstream secure API calls.

For the test environment coder

Request structure as follows:

TO BE ADDED

Response structure as follows:

TO BE ADDED

✅ Test Case 2.9.2: Use Case ST-0.2: Discover supported and available streams

Description / Purpose

This test case validates that the client can discover which streams types are supported and which actual streams and entities are available from the TAPI provider. The discovery typically includes the list of supported classes, available stream types, and operational parameters (e.g., encoding formats, supported paths). See Yang tree ST-02 and ST-02b for specific data nodes to be checked.

This step ensures the provider can support a client that can dynamically adapt to provider capabilities avoiding hardcoding stream definitions.

Prerequisites

- The client must have successfully obtained an access token (see Use Case ST-0.1) and is authorized to perform capability discovery.

- Provider should have initialized the appropriate streams (see Use Case ST-1.1).

Procedure

- Invoke the Capability Discovery API:

  - Send a GET request to the provider’s stream discovery endpoint.

- Parse each retrieved list entry (supported stream types and available streams) and identify relevant parameters of each stream instance.

- Validate parameters for correctness and consistency

- Capture details for later testing

for each available-stream including:

              - connection-address

              - connection-protocol

supported-stream-type-uuid as a key for the supported stream type parameters.for each supported-stream-type including:

              - Entities supported by the stream

              - log-storage-strategy

              - log-record-strategy

              - various parameters relevant to each specific test

Expected Results / Pass-Fail Criteria

| Step | Expected Outcome |  | Result |
| --- | --- | --- | --- |
| 1 | Request completes successfully with status code 200 OK |  | ✔️ Pass if request is accepted |
| 2 | Response includes key details required for operation of each supported stream (as required for later tests) |  | ✔️ Pass if data format and content is as required |
| 3 | Response for supported-stream-type indicates expected supported types and provides all necessary properties for operation (as required for later tests) |  | ✔️ Pass if data format and content is as required |
| - |  | Empty or malformed response, or incorrect schema, or any missing data as per criteria above |  | ❌ Fail |
| - | Not all streams expected are reported |  | ❌ Fail |
| - |  | Streams are not correctly described | ❌ Fail |  |

For the test environment coder

Request structure as follows:

TO BE ADDED

Response structure as follows:

TO BE ADDED

Test Case 2.9.3: Use Case ST-0.3: Connect to Stream and align - new client

Description / Purpose

This test case validates that a new client—without prior streaming history—can successfully connect to a stream and perform an initial alignment. This is a deep test of alignment of a chosen stream. Other streams will be validated with somewhat simpler tests.

Alignment ensures the client receives a consistent view of the target data set (e.g., current topology or state) before receiving incremental updates.

This is typically the first interaction after authentication and stream discovery.

This test is applicable to streams with either LOG_RECORD_STRATEGY_WHOLE_ENTITY

and LOG_RECORD_STRATEGY_CHANGE_ONLY  Log record strategy as reported in /tapi-common:context/tapi-streaming:stream-context/tapi-streaming:supported-stream-type/tapi-streaming:log-record-strategy

Prerequisites

- The provider must have initialized and be operating at least one stream (see Use Case ST-1.1).

- The client must have a valid access token (Use Case ST-0.1) and stream information (Use Case ST-0.2).

- There is no prior client stream state and is no resume token

- The network must remain operationally silent (so there will be no changes to data in the stream under test) during the test period.

- The stream must be stable and quiescent, i.e., no incremental updates occur during alignment test (as that complicates the test cycle).

Procedure

- Select stream for full testing

  - The alarm stream is proposed for this test. If the supplier does not support the alarm stream another stream can be used.

- Prepare a Clean Client Context

  - Ensure the client has no stored resume tokens or historical identifiers.

  - Initialize with default connection mode

- Send Stream Connection Request

  - The request will use the details from the available-streams and supported-streams responses from the earlier test

- Receive Initial Alignment Data

  - The provider sends contents of the log for that chosen stream

    - This will involve multiple records being sent

    - The flow of records will be stimulated by the test client behaviour

  - Note:

    - There may be tombstones in the log, these will be processed by the test client. Ideally no tombstones will be present at this stage

    - There may be gaps in the sequence numbers due to compaction. Ideally this will not occur as not compaction should take place as there is only one report of each entity

    - The test client will determine when alignment is complete

- Validate formation of each record

  - Examine each record to validate that it has the correct structure and key content

    - Sequence number increments (perhaps by more than one)

    - Time is sequential

    - All fields are supported appropriately

- Store the entities (and update with change as appropriate)

  - The test client will store records appropriately accounting for CHANGE_ONLY behaviour where relevant (although it is likely that the CHANGE_ONLY stream will only have initial records in during this test)

- Wait for Stream Quiescence (alignment complete)

  - Wait for a short, configurable period (e.g., 1–2 seconds) after the last streamed message.

  - This indicates that alignment is complete and no incremental changes are pending.

- Perform RESTCONF GET

  - Query the same object(s) using TR-547-compliant RESTCONF operations.

- Compare Both Results

  - Confirm that the streamed initial state and RESTCONF response are equivalent.

Expected Results / Pass-Fail Criteria

| Step | Expected Outcome | Result |
| --- | --- | --- |
| 1 | Client successfully connects to the stream (no auth or session errors) | ✔️ Pass |
| 2 | Provider sends complete initial state for the stream contents | ✔️ Pass |
| 3 | No new streamed messages are received for a short stability window (e.g., 2 seconds) | ✔️ Indicates stream quiescence |
| 4 | Client transitions to receiving updates (if the stream becomes active again) | ✔️ Optional Pass |
| — | Structure issues, content issues, stream incompatibility | ❌ Fail |
| — | Stream does not stop sending data | ❌ Fail |
| — | Structure issues, content issues, RESTCONF mismatch | ❌ Fail |

For the test environment coder

Stream connect structure is as follows:

TO BE ADDED

Stream record structure is as follows:

TO BE ADDED

For CHANGE_ONLY the record structure may only have a partial content as per the following example:

TO BE ADDED

Test Case 2.9.4: ST-0: Client maintains idle connection

To be detailed. Note that for gNMI there is no pong.

Test Case 2.9.5: ST-0.3: Connect to Stream and align - new client (all streams)

This has the same procedure as Test Case 2.9.3, but with no specific stream flow validation. The test is intended to validate stream content and that alignment is achieved other than the following considerations:

- The test case is repeated for each of the available streams

      - Alarms (P1)

note that there may be no RESTCONF comparison hence the alarms must be validated against those known to be in the network)

      - Topology (P3) As there is only one topology

      - Node (P2)

      - NEP (P2)

      - Link (P2)

      - CEP (P2)

      - Connection (P2)

      - physical route (P2)

      - SIP (P2)

      - connectivity services (P2)

      - Path (P3)

      - path set (P3)

      - Device (P2)

      - Equipment (P2)

      - Equipment Holders (P2)

      - Access Ports (P2)

      - PMs (P3)

with sufficient oam (with stated depth of coverage)

      - Oam (P3)

- (with stated depth of coverage)

The test will “drain” the stream and then compare the results with those from the corresponding RESTCONF GET.

Cross-Validation Note:
The data received during initial alignment must be structurally and semantically equivalent to the output of the corresponding RESTCONF GET operation defined in TR-547 (e.g., /tapi-topology:topology, /tapi-connectivity:connection, etc.).

This ensures the client can treat streamed state as a valid, eventually consistent substitute for traditional polling interfaces.

Why Quiescence Is Required:
RESTCONF GET and TAPI Streaming operate in different modes: RESTCONF is pull-based (snapshot), TAPI streaming is push-based (flow). To achieve observable eventual consistency between them, the network must remain stable during the test. We detect this stability by waiting for a short quiescent period (e.g., 1–2 seconds) after the last streamed message. This ensures:

- The alignment stream has fully drained.

- No concurrent changes are occurring that would cause drift between the stream and the RESTCONF view.

Comparison Method:

- Fields, structure, and values should match exactly.

- Field order is not significant (for JSON), but all fields must be present and correctly typed.

- Differences in encoding must be normalized for comparison.

For the test environment coder

Stream connect structure is as follows:

TO BE ADDED

Stream record structure is as follows:

TO BE ADDED

For CHANGE_ONLY the record structure may only have a partial content as per the following example:

TO BE ADDED